Emergency Response and Emergency Management Manual

The Emergency Response Manual has a number of functions:

- To provide an overview of HydroDive’s Emergency Response Protocol

- To Function as a Major Incident Manual

- To be the basses of all HydroDive’s Project Specific nd Site specific Emergency Response Plans

- The Emergency Management Section deals with how the business as a whole deals with an Emergency and how the business recovers from it, so it will include business continuity and mitigation of loss.

# SCOPE

This Manual covers the development of emergency response plans for operations off shore and remote from the base or jetty ( where they have their own standing procedures, though should a major incident occur then the structures in this manual may be appropriate)

This manual does not cover cyber emergencies not business continuity in the case of fire at a main base or the loss of a main base due to other unforeseen events.

|  | This document is NOT an emergency response plan in itself, the document out lines HydroDive’s Emergency response process and advise on the components necessarily included in a project specific emergency response. Parts of the manual may well be reproduced in this project specific response manuals The management section outlines HydroDive’s management process which is universal across the company – SOLAS and IMO requirement are covered by the Marine SMS though again a major incidents on shore management structure would be as detailed in this document, |
| --- | --- |

# How to use this document.

The principles in this document should be the starting point for the planning of emergency response plan for any operation, and the document shall be the reference point of all ERPs issued for HydroDive projects. Where it is felt that it is necessary to deviate the required MOC shall also trigger a review of this document to capture the cause of the MOC .

There are several symbols used throughout this document, this indicate certain actions or responsibilities these are:

|  | Use the decision-making model- (see HydroDive Supervisor Manual and Risk Management manual for in depth explanation) |
| --- | --- |
|  | Note – Critical Action |
|  | Risk Assessment Required |
|  | PUWER assessment required, (a risk assessment based around the Provision Use of Work Equipment Regulations). |
|  | IOGP Requirement |
|  | IMCA Requirement |
|  | Diving Medical Advisory Committee Requirement |
|  | Medical Requirement / or Emergency Requirement or Provision |
|  | Requires project Specific Information |

For the purpose of this document and in line with HydroDive risk management philosophy, risk shall be understood to the be the potential effects of uncertainty on a stated objective (Ref IOGP 510).

This document should be used in conjunction with HydroDive Risk Management Manual HDG-QMS-MAN-002

This manual provides HydroDive personnel with the necessary information, instruction and guidance to ensure both the relevant statutory requirements and those of our Safety, Health, Environmental and Quality Policy regarding Risk Management and safe and effective work planning are fully met.

All risks must be reduced to an acceptable level in line with HydroDive Risk model whether they pose financial, safety, health, environmental or quality impacts to the business.

The starting point of an emergency response plan is that Risk Management has failed, and an incident has occurred, and ERP is effectively a mitigation to ( usually0 a collection of fortuitously  aligned failures. Very few incidents have a single unique root cause.

This does not mean that now that the system has failed, risk management stops.  An emergency response plan is effectively an attempt to dynamically mitigate the uncertainties being generated from an unanticipated uncertainty, an example would be that the escape plans at PIPA ALAPHA did not allow for the fact that the escape routes might be in accessible, the same error could be found in the  Grenfell Tower conflagration.

| ISO 3100 definition of Risk | RISK = effect of uncertainty on objectives |
| --- | --- |

This document gives an overview of the Bronze Silver Gold structure and its implementation in Hydrodive for minor and major incidents. Project managers should use this structure when developing a ERP for their projects. It also outlines considerations that should be included and assessed and verified by the project mangers when creating ERPs. Things such as Ambulance routs, timings, hospital capacity , helideck capabilities should not be taken for granted unless verified.  Emergency vessel routes and timings should not be calculated off google maps by persons not competent to make marine transit calculations.

Firstly the Emergency response structure is the same across all HydroDive work site whether in Nigeria or outwith Nigeria on shore or off shore diving or non- diving, the structure remains the same.

# The Base Model

| ### Decision Making Models |  |
| --- | --- |
| Emergency situations are by definition unplanned events and whilst there is some predictability in the way that certain events will unfold lends it itself to the Recognition Primed decision- making process which makes the use of flow charts and procedures extremely effective in high- stress situations, where there are complications or simultaneous unplanned events it is essential that the critical decision makers have a decision-making model from which to function, even if that is only as to which flow chart to use, or where the situation has departed from any reasonably foreseeable event and urgent decision making is required. |  |
| A small version of this model will appear in the margins where the model is recommended |  |

|  | The following model will be employed by the three levels of command in the Hydrodive Emergency response process |
| --- | --- |

| ## Decision Making Model components |  |  |
| --- | --- | --- |
| Information – gather information and intelligence | During this stage the decision maker defines the situation (that is, defines what is happening or has happened) and clarifies matters relating to any initial information and intelligence. Decision makers could ask themselves the following questions. - What is happening? - What do I know so far? What do I not know? - What further information (or intelligence) do I want/need at this moment? |  |
| Assessment – assess threat and risk and develop a working strategy | This analytical stage involves assessing the situation, including any specific threat, the risk of harm and the potential for benefits. Develop a working strategy to guide subsequent stages by asking yourself the following questions. - Do I need to take action immediately? Do I need to seek more information? - What could go wrong (and what could go well)? What is causing the situation? - How probable is the risk of harm? How serious would it be? - Is that level of risk acceptable? - Am I the appropriate person to deal with this? - What am I trying to achieve? - Will my action resolve the situation? |  |
| Powers and policy – consider powers and policy | This stage involves considering the powers, policies and legislation that could apply in this particular situation. - Decision makers could ask themselves the following questions. - What authority level do I have in the company, What higher authority might be required? - Is there any industry guidance covering this type of situation? - Do any company, IMCA or IOGP or IMO policies or guidelines apply? - What legislation might apply? - Is there any research evidence |  |
|  | It may be reasonable to act outside policy as long as there is a good rationale for doing so. |  |
| Options – identify options and contingencies | This stage involves considering the different ways in which a particular decision can be made (or situation resolved) with the lowest risk of harm. The decision maker may need to consider: - what options are available immediacy of any threat - limits of information to hand amount of time available - available resources and support - your own knowledge, experience and skills - impact of potential action on the situation and the public a nd company reputation what action to take if things do not happen as anticipated |  |
|  | If you have to account for your decision, will you be able to say it was: - proportionate, - legitimate, - necessary - ethical? That it was reasonable in the circumstances being face at the time? |  |  |
| Action and review – take action and review what happened | This stage has two distinct steps. At the action step, decision makers are required to make and implement appropriate decisions. |  |
|  | Action | Respond - Implement the option you have selected. - Does anyone else need to know what you have decided? Record - If you think it is appropriate, record what you did and why. Monitor - What happened as a result of your decision? - Was it what you wanted or expected to happen? If the incident is continuing, go through the DM again as necessary. |
|  | Review | If the incident is over, review your decisions, using the DM. What lessons can you take from how things turned out and what might you do differently next time? If appropriate, evaluate the impact of the decision on outcomes. Have you liaised with individuals involved in the decision making process or affected by the decision? |

# Recording decision making

Decision makers are accountable for their decisions and must be prepared to provide a rationale for what they did and why. In some circumstances the need to document decisions is prescribed by statute, required by organisational strategies, policies or local practices, or left to the decision

maker’s discretion.

Whatever the circumstances, the HydroDive  recognizes that it is impossible to record every single decision and that not all decisions need to be recorded. In most instances professional judgement should guide decision makers on whether or not to record the rationale, as well as the nature and extent of any explanation.

The record should be proportionate to the seriousness of the situation or incident, particularly if this involves a risk of harm to a person.

If the incident is over, review your decisions, using the DM. What lessons can you take from how things turned out and what might you do differently next time? If appropriate, evaluate the impact of the decision on outcomes. Have you liaised with individuals involved in the decision making process or affected by the decision?

## Reviewing decision making

The DM can be useful for examining decisions made and action taken, whether by a supervisor or during an informal investigation or a formal inquiry. Demonstrable compliance with the model is a powerful mitigation to a allegation of negligence Reviewing decision making process is covered in the incident investigation manual- along with advice on understanding the process,

# HydroDive’s Emergency response Structure

To ensure that there is a coordinated response to any emergency, it is important to establish clear lines of communication. The emergency organization at the site of the incident will vary according to the location and nature of the incident but will, in all cases, be led by the OSC who will direct the onsite emergency services and be in direct contact with the EC and, if necessary, Hyperbaric Medical Centre, HydroDive Emergency Organisation includes.

For the purpose of clarity and ease responsibilities on this project should be allocated on a bronze Silver, Gold model as this is an internationally recognized model and easily synchronized with other organizations

## On Scene Coordinator (OSC) Bronze

The Offshore Construction Manager shall be the person designated as the Bronze Controller ( some tines referred to as the On Scene Coordinator- this manual will henceforth use Bronze Silver and Gold). He shall initially establish contact with the Silver  Coordinator (previously referred to as Emergency coordinator ) and the Client On- Scene Coordinator (COSC). In a circumstance where the Bronze Controller is the injured person, then the person considered most likely to be capable of responding to the emergency will be the vessel master or Diver Medic ( the ERP should risk asses this and ideally a person shall be identified as the most appropriate reserve Bronze Controller – see planning section of this document).

The Bronze Controller will be responsible in conjunction with the vessel medic

## Silver Coordinator (SC)

This is a member of senior management onshore. HydroDive Emergency Coordinators in project situations are usually as follows:

      - Project Manager

      - Designated Person Ashore

      - Marine Manager

The Silver Coordinator   will be the focal point for the entire coordination of the emergency and as such will be the initial point of contact with the OSC. The EC will determine the level of response required and will manage the response effort.

The Silver  will remain in contact with the COSC (after the initial contact between the Bronze Coordinator and COSC) at all times during the emergency, updating him on the progress of events and receiving direction as necessary.

This communication allows client and HydroDive senior management to manage the strategic elements of the emergency response e.g. Medevac, while Silver Coordinator will manage the short and medium-term response of the HydroDive personnel and assets.

A Duty Roster shall be maintained for the Silver Coordinator. The SC who is on duty at the time of the emergency shall continue to act as the Silver Coordinator until the emergency has been resolved. If there is a specific skill required if may be deemed appropriate by the Gold Coordinator to appoint a Discipline Specific Silver coordinator  ( e.g. in a maritime Emergency The Marine Manager may be better equipped to fill the function, than the Dive Manger or Project manager)

It is also the responsibility of the Silver Coordinator in conjunction with the COSC to decide when to stand down the ECT. The Silver Coordinator is responsible for issuing a report to the HSE Manager of how the Emergency Response was  handled  and lessons  learnt  as  soon as possible after the emergency.

## Gold Controller (GC)

The Gold position In Hydrodive is the strategic Lead. They hold responsibility for the handling of the incident and will set an overall strategy for dealing with it. This will include any tactical do’s and don’ts that need to be followed. The Gold Leader should not be drawn into making tactical decisions. Their focus stays at a strategic level however they will be responsible for reviewing and ensuring that the tactics being utilized are appropriate and sufficient to meet the strategic goals that they have set.

Some of their responsibilities will include:

- Assuming overall command and control of the incident

- Set, review and communicate their strategy

- Handling the media strategy

- Report to shareholders if appropriate

- Appoint someone to act as Chair for the Silver Team

- Consider the community and wider impact of the incident

- Define what resources or specialist skills are needed

- Appoint from The Emergency Team the Most appropriate Silver Coordinator for the Situation



## Emergency Coordination Members (ECM)

Emergency Coordination members include other members of HydroDive Management who are relevant in emergency situations. They are as follows:

| MD/CEO | Is the de facto GOLD Manager with overall responsibility for the incident till final close out at a corporate level |
| --- | --- |
| Diving Director | May be the delegated Gold Coordinator- or assume some delegated responsibilities or be appointed Silver controller by the GM |
| Personnel Logistics Manager | Responsible for logistics control in country and repatriation also involved with Family liaison for expatriate workers Where there are logistics and equipment concerns or the need to mobilise such will provide support to the Silver EC |
| Human Resources Manager | Responsible for authorizing and coordination of Medical Treatment on shore |
| Marine Manger | Senior advisor would be Silver Controller in Marine Incident and an advisor on marine capacities where needed to support another EC in incident management |
| Project Manger | - Assuming overall command and control of the incident - Set, review and communicate their strategy - Handling the media strategy - Report to shareholders if appropriate - Appoint someone to act as Chair for the Silver Team - Consider the community and wider impact of the incident - Define what resources or specialist skills are needed |
| Legal | To Advise the Gold Controller on the legal ramifications of the emergency and the implications to the company and the company’s legal responsibilities – enabling the Gold Controller to make the best decisions in the Company’s interest |

## Emergency Coordination Team (ECT)

The ECT includes all key players in an emergency situation. It consists of the Bronze Coordinator Silver Coordinator, HSE Manager, COSC (and may include  Hyperbaric Medical support  Facility (ISOS)).

## Hyperbaric Medical Centre

If the Bronze requires expert hyperbaric medical assistance, he will first make

contact with ISOS (HydroDive’s Hyperbaric Medical Centre) and thereafter ensure that the on-call doctor liaises directly DS with the Bronze always keeping abreast with developments.

## HSE Manager

The EC will contact the HSE Manager who will be responsible for notifying and updating the Chief Executive Officer at the earliest opportunity consistent with the ultimate priority of resolution of the emergency. The HSE Manager will provide the EC with advice and any assistance the EC requires as well as maintaining a initiate the Accident & Incident and Close-out Report.

The HSE manager will be responsible for maintaining and up to date real time running log of the incident until it is decreed by the GM that the incident has been closed. These notes will assist in the close out and root cause analysis and any evaluation of the efficiency of

the response and be used for lessons learnt going forward. These will be a legal document and constitute original notes made at the time.

## Legal

To Advise the Gold Controller on the legal ramifications of the emergency and the implications to the company and the company’s legal responsibilities – enabling the Gold Controller to make the best decisions in the Company’s interest. The Legal team member will also vet all communications with the press and outside parties prior to release

## Client-On-Scene Coordinator (COSC):

The Client  will, when circumstances dictate, appoint their own COSC who will direct the onsite emergency services e.g. medevac. He will be in direct communication with the HydroDive Bronze Coordinator and Silver Coordinator . Under normal circumstances, the Client Site Representative (CSR) is usually the COSC.

In all circumstances, it is important that there is a nominated COSC offshore at all times and that his identity is known to all personnel and other parties involved in the response to an emergency. The COSC will be in charge of mobilizing the Duty Doctor and Aviation support from Shell

## On Site Medical support

Both HydroDive DSV’s have a qualified medical doctor on board who will be able to assist in triage and provide stabilization of trauma related casualties. The goal being to maintain MEDIVAC status and transport a stabilized casualty.

HydroDive personnel are trained in Basic First Aid.

|  | The ERP designer cannot assume the presence of qualified personnel on board, also the fact that the doctor might be the casualty should be considered. |
| --- | --- |

Where it is a diving operation there will be at least two DMT’s per shift and all diver’s and supervisors will hold an in-date Advanced First Aid ticket.

# Emergency Management Off Shore

      - Immediately after an incident has been identified a primary assessment shall be made as to the type(s) and severity of the incident.

|  | This classification of the incident will be fluid and may escalate rapidly, and the Bronze EC must be prepared to change his assessment and aware of the risks of confirmation bias and denial, in his ongoing assessment. A marine incident resulting in vessel incapacitation and loss of productivity may be concurrent with serious injuries to crew members |
| --- | --- |

Thorough investigation and assessment by the first person (supervisor) on scene are essential and clear reporting.

|  | Immediate logging of the incident(s) should commence the bridge log will at this stage be an appropriate legal document into which detail may be recorded before a separate incident log is commenced, in this instance, the record originally made in the bridge log will for legal purposes constitute original notes, but may continue into the designated incident log- which will reference the bridge log. |
| --- | --- |

The four-tired response system is a useful decision-making tool when assessing the level of intervention required

# Incident Gradings by Tier at a Company Level

| TIER 00 | Business as Usual | The incident has little or no impact on the ability to deliver the project. This level of incident would be covered by a first aider and would require mitigations such as a safety flash, revised risk assessment or a time out for safety. |
| --- | --- | --- |
| Tier 1 | Tactical Response | This is a Front Line level Response to protect People and Property and relies on the on-site team medic and DMT s This will result in the some disruption but can ordinarily be managed aby the on scene Bronze Level Controller and the project deliverable will be able to continue with minor disruption There will be a requirement foe on Shore HSE intervention and Lessons learnt implementation |
| Tier 2 | Operational Response | This level of situation will adversely affect project delivery and require significant input from Silver Level Control. This type of situation may include mechanical incidents that prevent the company from working. Incidents that are High Potential These will be addressed by procedural review, further training Lessons learnt |
| Tier 3 | Strategic Response | This are life threatening incidents or operational incidents that seriously threaten the vessel, project or company and require full managerial response Thes incidents will absolutely require a full mobilization of the management response team and require significant input from the Gold Level Controller |

Where there is a casualty involved, the tier response on the scene is also used as a micro version of the above, these are influenced by timing and casualty management.

# Incident Gradings by Tier on scene

At the scene, the Tier system is also used

| TIER | Description | Min Response Time | Assigned Party | Contact Details |
| --- | --- | --- | --- | --- |
| 0 | First response by people at the emergency site | Immediate | Any of the first aid trained divers. One of the two Diver Medical Technicians (DMT) on shift. | Update names DMT on each shift. Contact by PA |
| 1 | First Aid on site | 4 minutes | One of the two Diver Medical Technicians (DMT) on shift. Ships Doctor | Name Doctor Contact by PA; (room number. Internal vessel phone) |
| 2 | Patient assessment and commence treatment by site doctor / Nurse / Paramedic | 1 hour | Ships Doctor Onshore medical support. | Name Doctor Contact by PA; room number. Internal vessel phone. Name and number project onshore medical support. |
| 3 | Referral to appropriate specialist hospital | Casualty Specific. | EC and COSC initiate onward medivac to Lagos | Retainer Clinic PH:/ LAGOS / Warri or as appropriate to the location of the project |

# Initial Response

Initial response will be determined by the incident, much of which is covered by the vessel’s own process.

However, the primary actions:

  - Raise Alarm

  - Assess and evaluate

  - Make safe the scene before attending to casualty(s)

  - Assess casualty(s)

  - Triage

  - Where necessary move casualties to safe space

  - Treat on site

  - Stabilize

  - Document

  - The on-scene EC (Bronze) will notify the EC (Silver) as soon as possible (who will commence the on shore log and take the actions noted in the following section (Emergency Management ON SHORE )

# Emergency Management  ON SHORE

As soon as the duty Silver Emergency Controller is notified of an incident, based on the Bronze EC’s assessment he/she will make a decision as to the level of response required. This may be escalated or deescalated as the incident develops.

The first action will be to notify and assemble the Emergency Response Team. The designated team members will attend the Hydrodive main building in Apapa Lagos. The designated Emergency Response room is the Conference Room on the Management Floor of the main building

This room is equipped with phone lines, white board, Internet connection, consol based speaker phones, and a large monitor screen, Video conferencing facilities are also available.

Once the team is assembled and the initial situation understood the Gold Controller will either confirm in place the EC Silver or if there is a discipline available more appropriate to the situation will appoint that person as EC Silver for the duration of the situation

As soon as the response room is established a running log will be maintained by the HSE lead. All decisions will be recorded in the proper format, verified by the legal representative. The GM (or his Director of operations as Gold Controller) will have the final say in the decision-making process

It is essential that no communication leave the room without the verification of the EC Silver. All Press communications should be run through the Legal representative and verified by Gold EC.

Where there are fatalities or serious injuries communication with the family will be via HR with the assistance of Legal.

|  | Hydrodive will impose a social media blackout on all failure to observe this blackout will be considered a serious disciplinary offence |
| --- | --- |

      - .

## REFERENCES

| Document Title | Document No. |
| --- | --- |
| Project HSE Plan | HDG-PRJ-HSE-863-03 |
| Project Mobilization Plan | HDG-PRJ-MOB-863-01 |
| Health, Safety & Environmental Manual | HDG-HSE-MAN-001 |
| Accident & Incident Reporting | HDG-HSE-PR-001 |
| Diving Operations Manual | HDG-DIV-MAN-001 |
| Diving Emergency Flow Chart | HDG-HSE-FRM-022 |
| IOGP 411 | Jan 2021 |
| IOGP 478 | Jan 2018 |
| IMCA D014 – International COP | Rev 3.2 May 24 |
| IMCA D022 – Guidance for supervisors | Rev 1.3 Jun 24 |

# EMERGENCY ORGANIZATION & COMMUNICATION

## Emergency Communications

It is essential that in establishing the initial contact and in all subsequent communications, precise details of the incident are passed using the format shown below. In the event of an Emergency, it is imperative that the OSC (the OCM – details provided in Appendix 1) provides every relevant detail to the EC & DFT to ensure a complete and appropriate response.

In all cases where any doubt exists as to the seriousness of a situation it is important that early contact is established if only to solicit a second opinion.

## Employee Orientation Procedure

All new HydroDive employees will be required to attend an orientation program prior to employment, which will, as part of its overall content, address elements of the emergency response procedure. All project personnel shall undergo a site/vessel induction course.

## Visitor Orientation Procedure

All visitors to the Project site will be required to go through an induction process and sign a form that they have read and understand the Visitor Orientation document. The visitor orientation will address elements of the emergency response procedure

### Incident  Reporting

|  | Note here ‘Incident Reporting’ refers to the passing of information to the necessary parties in odder to take immediate action – this is distinct from the ‘incident Reporting Process for QAQC purposes and subsequent investigation which is covered the HydroDive incident Reporting Manual++++++++++ |
| --- | --- |

The following format is to be used in passing details, whatever mode of communication is used:

TIME AND DATE OF ACCIDENT; TYPE OF ACCIDENT:

PLACE OF ACCIDENT:

NAME(S) OF PERSONNEL INVOLVED:

WHAT IMMEDIATE ACTION HAS BEEN TAKEN INCLUDING ANY ADMINISTERING OF MEDICINES ETC:

ADVISE BEST MEANS OF COMMUNICATION: I.E. RADIO, TELEPHONE (HARD LINE/SATELLITE), FACSIMILE:

ANY OTHER RELEVANT DETAILS

Incident reporting and investigation will be under SPDC HSE MS. ( Type 2 contract)

Once the Project Manager – (details provided in Appendix 1) receives the initial report above it will be his sole responsibility that he and the ECT (details of the ECT provided in Appendix 2 – Emergency Flow Chart) complete a full investigation and close out the emergency.

      - Do everything necessary to save life, and prevent damage/loss of property and environmental pollution.

      - Maintaining a full record of the progress of the emergency and the response effort

      - Organization of marine, sea and land transport medevac liaising with Shell PMT team

      - Arranging hospitals, clinics, overseas medevac and Hyperbaric Medical cover where appropriate

      - Liaison with the COSC

      - Answering telephone enquiries from relatives of personnel involved in the emergency.

      - Production of list of next of kin of personnel involved at the emergency site;

      - Liaison with Local Government services; Police, Fire Brigade, Ambulance Service etc., at the direction of the EC;

      - Answering telephone enquiries from media, government and other external parties at the direction of the EC;

      - Preparation of holding statements and press releases for screening by the CEO

      - Liaison with the Chief Executive Officer as required.

      - Complete a full investigation and close out of the emergency

# HydroDive Interface with Client Organization

Close co-operation between HydroDive and client Management will be essential in any emergency. This co-operation should include but not be limited to the following:

- Immediate and long-term operational response to the emergency, with the objective of minimizing and if possible, avoiding: -

- Injury and loss of life to personnel.

- Harm to the environment.

- Damage to physical assets.

- Damage to intangible assets such as corporate image and reputation.

- Investigation of the cause of the emergency

- Release of information concerning the cause and all other aspects of the emergency.

# MEDEVAC & CASEVAC (Medical Evacuation & Casualty Evacuation)

Often, the terms CASEVAC and MEDEVAC are used interchangeably, but understanding the differences between these terms is crucial for effective communication and coordination during emergencies.

|  | A “MEDEVAC” is defined as the evacuation, in emergency or not, of one or more, injured or ill personnel to a hospital onshore where further treatment will be given |
| --- | --- |
| A Medevac from a worksite comprises: - Work location to port/heliport – route confirmed and means of transport. - Fast Boat and Onshore or helicopter transportation between site and a clinic. - Repatriation: when necessary, managed with the assistance of the company |  |  |

.

|  | CASEVAC is short for casualty evacuation |
| --- | --- |
| This method of transport conveys injured persons to a medical facility. These evacuations are driven by urgency, so they are conducted by the closest team regardless of their medical background. A CASEVAC requires immediate action to get individuals to safety, while a medevac allows for a little more time to plan and get organized. |  |

Generally, the most likely envisioned scenarios covered by this procedure will be MEDEVAC with the casualty stabilised by the on-vessel medical team (Medic and DMTS) prior to transport.

There may however occur situations where the risk of delaying transport outweighs the risk of the transit itself, in this instance the Vessel Master in conjunction with the Medic, if both are survivors would make the call to upgrade to a CASEVAC scenario.

The most likely scenario for this type of escalation would be a multiple casualty situation exceeding the capacity of the vessel’s medical facilities.

|  | In the event of a multiple casualty scenario, a decision can be made by the Vessel Master to initiate the vessel ‘Mayday’ Protocol or ‘Pan Pan’ this is a serious step to take and as this is not a Maritime Manual the outlines only of the decision-making protocol will be outlined here as the client may not be able to ensure sufficient Helicopter response in an appropriate time scale and other agencies and protocols may be invoked ( e.g. SOLAS) |
| --- | --- |

The Master and OCM are referred to the earlier depicted decision-making model – which should be maintained as the situation escalates or deescalates

Pan Pan

Pan Pan is used to signify a situation that is urgent but not immediately life-threatening. It indicates that a vessel or aircraft requires assistance but is not in immediate peril. This might include technical issues, crew illness, or equipment failure. Using Pan Pan alerts others to stand by for further communication or assistance.

Mayday

Mayday is reserved for grave and imminent danger requiring immediate assistance. It is the most urgent distress call. Situations involving sinking, fire, or critical medical emergencies fall under this category. When Mayday is declared, it indicates that those on board are in immediate danger, and help is crucial

# Depending on the severity of the incident, the appropriate medical response would be initiated.

In the event of a casualty situation occurring and evacuation being required, HD would initiate their ERP, concurrent to the client  being informed of the immediate support requirement. The client would be contacted as per the contact details supplied in this delivered and approved project ERP  concurrent to the ERP being initiated by the HD OCM (OSC) on the advice of the Medic onboard / HD Contender.

# Primary and secondary means of evacuation identification ad selection

|  | When Planning the ERP the designer should identify a primary and secondary means of evacuation ( there may even be a tertiary if the risk assessment dictates |
| --- | --- |

The designer needs to start with the objective of  evacuation a casualty ( medivac) as safely, quickly and efficiently to a place where ongoing medical treatment can occur

There may be a number of modes of evacuation available   the decision as to primary and secondary preference may not be immediately obvious untill the options are assesed side by side.   Here is an example

| Helicopter |  |
| --- | --- |
| Advantages | Disadvantages |
| - Availability - Speed - Has options for landing - Can bring appropriate medical team to provide life support before and during evacuation | - Availability - Heavily weather dependent - May not be able to land - May be limited in multi casualty situation - Landing area may require long road transport to hospital or treatment location - There may be height (max and minimum) restrictions especially important in hyperbaric casualty evacuation - Will be a two way journey - May need to mobilise staff from titiary location - May be air borne at time of call and need to land to off load passengers before attending emergency site |

| Surfer Boat |  |
| --- | --- |
| Advantages | Disadvantages |
| - Availability generally on site as safety boat and infield transport so no out ward journey to account for - Can accommodate multiple casualties and generally there will be space for ongoing treatment - Different weather limitations to a helicopter | - Availability - Heavily weather dependent - May not be able to land - May be limited in multi casualty situation - Landing area may require long road transport to hospital or treatment location - There may be height (max and minimum) restrictions especially important in hyperbaric casualty evacuation - Poor sea sstate can lead to rough transit inappropriate for spinal chord injuries or broken bones - Possible berthing area rwuires greater road on ward road transport to hospital |

The above is just an example an it is important that the geographical factors are taken into account

For example, when assessing a surfer boat transit time it is common for designers to simply take a top speed of the vessel measure the distance of the trasit and produce a transit time.  This is tyotally in appropriate ideally the calculation should be made by a person with navigation understanding and account for tidal streams in the area  the calculation based on the fastest point of the tidal stream on a spring tide for that area, done correctly it may be found that the anticipated trasit time is quite unfeasible.

If an escort is required, the calculation is based on the slowest vessel

Thedesigner should also consider the availability 24hrs of port staff to receive the vessel’ customs and immigration requirements all of which could add considerable to  the transit time

Whatever method is selected The designer than needs to consider the on shore transit to the destination

It should be clearly identified in the Erp who is supplying and coordinat0on g the on shore transport. The transit time for wherever they are located to the gheliport or dock and the transit time to the hospital orhyperbaric facility.]

The designer should have verified that the ambulance proposed are suitably equipped with staff and equipment to make the transfer

The Designer must consider that the ambulance will not be on permanent standby for his project and must verify that the supplier has  enough ambulances to deal with multiple casualties and canhas enough capacity to deal with their normal work load and the HydroDive evacuation.

The designer must verify the road root planned and the timings and consider seasonal variations that might make the roads impassable and identify alternative routs that would not be affected under the same conditions , these should be time estimated allowing for traffic and weather.

# Decision making to switch to secondary

The ERP should include a decision making guide to assist the Bronze and Silver Controllers  based on potential failure of the Primary Evacuation mode

This is an example od such guidance but should be project specific

|  | Life threatening injuries Trauma Amputation Sever Blood loss Multiple casualties Cardiac Arrest Stroke Critical Illness | 10 minutes from request for availability to receive confirmation that a helicopter is available – Then doctor and OCM can decide to mobilse option 2 |
| --- | --- | --- |
|  | Serious non-life-threatening Illness Psychotic episodes Mental illness | 40 minutes |
|  | Illness or injury rendering un fit to work requiring on-shore medical attention | 1 hour |

# Reception

Hospitals should be identified and physically visited by a HydroDive representative and  the designer should have made all reasonable steps to ensure that the hospital has a capacity and capability to deal with a variety of trauma related injury. The hospitals capacity for multiple casualties should also be assessed and noted.

There must be clear contractual arrangements tat are in date

|  | The designer should assess whether or not an alternative Hospital needs to be included on the ERP |
| --- | --- |

# Hyperbaric reception

On shore hyperbaric reception facilities need to be identified, these should have in date DO23 in place and they should be regularly audited by Hydrodive, there must be a clear contractual arrangement in place between HydroDive and the Supplier ton ensure that the Hyperbaric facility will be available and maintained in date.

The designer should also take the same precautions with road transport to the facility abd tinings and alternative routes. The availability of staff at the centre nd whose responsibility it is to supply them should also ne noted

# ON SHORE HYPERBARIC MEDICAL SUPPORT

The ERP must contain for all diving operations a designated diving doctor support system, this is usually an organisation not an individual, the ERP designer shall ensure that the contract in place ad maintained an that the supplied numbers are correct and that where the organisation requires to be notified at the start of a diving project that this is done.

The ERP must also identify that the Hyperbaric Doctor can be contacted, and a stable construction ,obtained , be this by Sat Phone Mobile Phone Radio, a single means of communication is not sufficient as  it may be necessary to remotely  direct intrusive medical intervention, if the connection were to fail and not be established during such a process the consequences coud be catastrophic

Where Primary means of transporting an IP ashore would be by helicopter from a hydroDive managed vessel

HydroDive shall ensure that the required Helicopter landing support competent persons (Helideck Team- HLO, HDA/HERTL/HERTM) are always onboard the vessel – this shall be included as part pof the ERP

| HLO | Helicopter Landing Officer |
| --- | --- |
| HDA | Helideck Assistant |
| HERTL | Helideck Emergency Response Team Lead |
| HERTM | Helideck Emergency Response Team Member |

Again redundancy of support should be confirmed with marine.

|  | A helicopter incident on the vessel helideck is not withing the scope of this planned ERP manual, as stated elsewhere in this document the onshore management structure would be as outlined above but the Bronze level management would fall to Marine personnel bd the response directive to the Marine Safety Management system |
| --- | --- |

In the event of unavailability of Medevac flight e.g., during bad weather or in the absence of a helicopter. The secondary Medevac means shall be utilised.

In mmany instances the client will not havd a designated rescue helicopter ( especially during daylight hours) and  it may be necessary to divert an already airborne helicopter,in this instance  the secondary response will always be activated simultaneously as  in case the Helicopter arrival time cannot be guaranteed,

It should also be factored that during dayligt hours and normally offshore flight operating times   that a helicopter that is on ground base that needs to carry a ERP team may need to collect an ERP team before leaving for site, this generally happens at the same time as the Chopper is being readied for movement to site, With the seats being removed. Any necessary road moevemnts to support this should be factored in to the potential arrival on scene time.

| Life threatening injuries Trauma Amputation Sever Blood loss Multiple casualties Cardiac Arrest Stroke Critical Illness | 10 minutes from request for availability to receive confirmation that a helicopter is available – Then doctor and OCM can decide to mobilse option 2 |
| --- | --- |

In the event of a multiple casualty scenario, a decision can be made by the Vessel Master to initiate the vessel ‘Mayday’ Protocol or ‘Pan Pan’ this is a serious step to take and as this is not a Maritime Manual the outlines only of the decision-making protocol will be outlined here as SPDC may not be able to ensure sufficient Helicopter response in an appropriate time scale

The Master and OCM are referred to the earlier depicted decision-making model – which should be maintained as the situation escalates or deescalates

Pan Pan

Pan Pan is used to signify a situation that is urgent but not immediately life-threatening. It indicates that a vessel or aircraft requires assistance but is not in immediate peril. This might include technical issues, crew illness, or equipment failure. Using Pan Pan alerts others to stand by for further communication or assistance.

Mayday

Mayday is reserved for grave and imminent danger requiring immediate assistance. It is the most urgent distress call. Situations involving sinking, fire, or critical medical emergencies fall under this category. When Mayday is declared, it indicates that those on board are in immediate danger, and help is crucial

### Depending on the severity of the incident, the appropriate medical response would be initiated.

In the event of a casualty situation occurring and evacuation being required, HD would initiate their ERP, concurrent to SPDC being informed of the immediate support requirement. SPDC would be contacted as per the contact details supplied in this plan, concurrent to the ERP being initiated by the HD OCM (OSC) on the advice of the Medic onboard / HD Contender.

Primary means of transporting an IP ashore would be by helicopter via Contender Helideck. IP will be - NCAA, Shell Aviation and Bristow prior to being used on the project. HydroDive shall ensure that the required Helicopter landing support competent persons (Helideck Team- HLO, HDA/HERTL/HERTM) are always onboard the HD Contender. For the Forcados scope the following team applies:

| HLO | Helicopter Landing Officer |
| --- | --- |
| HDA | Helideck Assistant |
| HERTL | Helideck Emergency Response Team Lead |
| HERTM | Helideck Emergency Response Team Member |

In the event of unavailability of MEDEVAC flight e.g., during bad weather or in the absence of a helicopter. The secondary Medevac means shall be utilised.

As it has been identified by SPDC that there is no designated rescue helicopter and that it may be necessary to divert an already airborne helicopter, the secondary response will always be activated simultaneously in case the Helicopter arrival time cannot be guaranteed, it is also anticipated that a helicopter that is on ground at NAF base that needs to carry a ERP team will need to transfer to Shell RA before leaving for site, this generally happens at the same time as the Chopper is being readied for movement to site, With the seats being removed. The road move has been assessed as a drive of 15 to 20 min.

| Life threatening injuries Trauma Amputation Sever Blood loss Multiple casualties Cardiac Arrest Stroke Critical Illness | 10 minutes from request for availability to receive confirmation that a helicopter is available – Then doctor and OCM can decide to mobilse option 2 |
| --- | --- |
| Serious non-life-threatening Illness Psychotic episodes Mental illness | 40 minutes |
| Illness or injury rendering un fit to work requiring on-shore medical attention | 1 hour |

Where it is confirmed by SPDC that a helicopter is in the air and en-route communication shall be established that will allow the target vessel to steam a course to establish an RV point that bisects the tow trajectories to minimize the return flight time as much as possible.

Where there is no confirmation of a helicopter in the air and the above time limit for aborting primary and reverting to secondary response it should be considered that a vessel underway will still shorten the response time. This is assuming that it is not a marine incident where the vessel is incapacitated

# Devising an emergency  project specific Emergency respons plan

The  previous section out lines the HydroDive Emergency response structure that will manage a response bit each project must have its own unique plan that complies with the structure.

As steted earlier in this document the activation of an ERP means that the existing safety mitogations have failed, so all action is bessed on a level of uncertainty, a good ERP will bring back a level of certainty to the disorder.

One of the first things to establish is what are the scenarios to which we coud be responding, a good place tp start id the Project HIRA, and look at all the identified risks we know these can happen – we mitigate against them – the barrier has vfailaed. The Project HIRA should give the Erp designer a base line of possible scenarios and casualty numbers  and types of injury on which to base his design.

On a somple level an air diving operation on a DSV holdees some quite differebt scenarions from a Sat diving operation on a barge.

In all the planning use the 31000 model – here is an example:

First place to start is with the Bronze level response,  before we start designing the bronze response the Bronze commander has to be identified trained and available to take control of the situation

First Uncertainty – the Bronze commander is the casualty – or amongst the casualties.

Mitigation – two persons must be available to take the role so a second competent person should be identified and trained to be the Bronze Controller.

|  | Here an important policy decision must be made by the designer, it is recommended that the decision taken at this point be applied through out the process; How many steps of mitigation are taken This means do we mitigate against the failure of a mitigation- generally it is rare to go more than three steps, but there are measures we can take to make the mitigation more robust. Using the example of the Bronze commander being a casualty we have identified a second person to step in , we now make a choice we either appoint a third in the case that both Commander A nd Commander B are injured or we try to ensure that they are NOT both casualties, a solution could be to have A on day shift B on night shift A in a starboard Cabin B in a port cabin, this is the same principle as Airline pilots on the same flight not being allowed to eat the same meal. This is a choice to be made by the ERP designer based on the vessel project and circumstances. It might be noticed that this is a similar philosophy to the design of a DP 2 system vs a DP 3 system – having more redundancy and independence vs making the separation and defence of each redundancy greater. |
| --- | --- |

The ERP Designer shall then consider the resources available to the Bronze Controller at the scene, such as but not limited too:

- Medical facilities

- First responders

- Firefighters

- Security personnel

- Communications sytems

- Life rafts

- Stretchers

- Manpower

- Lighting

- Fuel

- shelter

Whatever is relevant to the scenario being considered

A frequent flaw in ERP planning is the multiple casualty scenario, minor incident escalate rapidly to major ( Grenfell Tower a poor plug socket on a fridge lead to 77 fatalities, Challenger Space Shuttle a single ‘O’ ring failure lead to 7 fatalities, Mumbia High , cut finger in kitchen 22 fatalities  ).

There is a noticed tendency despite the legacy of Pipa Alpha and Deep-Water Horizon to treat ERP planning as a routing tsk and plan for the ‘best disaster’ , this is a serious concern and a ERP designer following this process should avoid that pitfall

|  | An ERP is not the minimum requirement to meet client and regulatory demands, it is a demonstration of the moral and ethical integrity of the company and the backbone of the safety culture |
| --- | --- |

The ERP Designer should based on the scenarios envisage identify the gaols and objectives of the Bronze Commander- Standard phrases such as ‘ secure the scene’ and ‘ preservation of life’  and ‘ Prevention of escalation’ and ‘containment’  are  useful triggers but should not be used as a substitute for thinking and analysis and understanding.

It's useful to look at ranking these

Generally the emotive response is ‘preservation of life’ - this response frequently leads to escalation.

Example – unconscious casualty in confined space – rescuers rush in, we now have two unconscious casualties to recover.

Whilst it reads as counter intuitive a more realistic start point is containment.

- Containment

- Prevention of escalation

- Preservation of life

- Secure the scene

|  | The ultimate goal is PRESERVATION OF LIFE but that is frequently only achievable by following the steps |
| --- | --- |

The ERP designer is not expected to envisage and document every conceivable combination of events but should consider the most likely and then the path of response and the goal of that response should be identified.  Then the variable that could affect that goal identified and mitigated.

# Planning evacuation options

It is  worth here revisiting the Hierarchy or teiered response leverls and remembering that a teir 00 can repaidly escalate to a teir 3 response, also the  evacualtion options should not add to the risk and be demostarbley viable.